TABEL JURNAL

Nama: M Yusuf Rifa Afriza         NIM: 607022400011

| AUTHOR | JUDUL | TAHUN | METODE | ALGORITMA | ALAT | KEKURANGAN | KELEBIHAN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Krebs, S. & Herter, T. (2024) | Ultra-Wideband (UWB) Positioning System Based on ESP32 and DWM3000 Modules | 2024 | Rancang bangun / prototipe | Two-Way Ranging (TWR) + Extended Kalman Filter (EKF) di sisi tag | 6 board ESP32 + modul DWM3000 (Qorvo); 1 tag, 5 anchor | Jumlah anchor tetap; posisi anchor harus diketahui a priori; NLOS tidak dibahas khusus | Akurasi hingga 10 cm; EKF berjalan lokal di tag sehingga tidak bergantung server; arsitekturnya paling mirip dengan proyek ini |
| Shalihan, M., Liu, R. & Yuen, C. (2022) | NLOS Ranging Mitigation with Neural Network Model for UWB Localization | 2022 | Eksperimen + pembelajaran mesin | Neural Network (probabilitas LOS) + Weighted Least Square (WLS) | Modul UWB low-cost (tanpa data CIR) | Perlu data latih untuk setiap lingkungan; hanya memanfaatkan kuat sinyal | Akurasi naik 16,93% (lobi) dan 27,97% (koridor); tidak memerlukan CIR |
| Angarano, S. et al. (2021) | Robust Ultra-wideband Range Error Mitigation with Deep Learning at the Edge | 2021 | Deep learning + eksperimen | Representasi CIR (CNN) + optimasi graph | Transceiver UWB dengan akses CIR | Memerlukan CIR mentah; beban komputasi lebih besar | Koreksi galat NLOS/LOS di sisi edge; ringan untuk perangkat embedded |
| Fan, R. & Du, X. (2022) | NLOS Error Mitigation Using Weighted Least Squares and Kalman Filter in UWB Positioning | 2022 | Simulasi + eksperimen | WLS-RKF: uji hipotesis Mahalanobis + Kalman filter | Testbed UWB positioning | Perlu penyetelan ambang; bergantung pada model gerak | Tidak memerlukan pengetahuan awal distribusi NLOS; akurasi 5 cm |
| Bregar, K. (2023) | Indoor UWB Positioning and Position Tracking Data Set | 2023 | Pengukuran / penyusunan dataset | ADS-TWR, multilateration (least squares) sebagai baseline ML | DWM1000 + STM32F103 + Raspberry Pi 3; komunikasi MQTT | Hanya berupa dataset, bukan sistem real-time; memerlukan praproses | 8 anchor, 4 lingkungan indoor, menyertakan CIR + ground truth laser; memakai MQTT seperti proyek ini |
| Yao, L., Yao, L. & Wu, Y.-W. (2021) | Analysis and Improvement of Indoor Positioning Accuracy for UWB Sensors | 2021 | Model matematis + simulasi MATLAB + eksperimen | EKF; model galat 2D/3D; distribusi skew-t untuk kondisi NLOS | DWM1000 + STM32F0/F4 + IMU MPU9250 + Raspberry Pi 3 | Butuh minimal 3 anchor (2D) atau 4 anchor (3D); model cukup kompleks | Memberi panduan penempatan anchor optimal; EKF unggul saat data mengandung outlier |
| Kramaric, L., Mustra, M. & Radisic, T. (2025) | The Influence of Ultra-Wideband Anchor Placement on Localization Accuracy | 2025 | Eksperimen (4 konfigurasi anchor) | Trilateration DS-TWR + perhitungan PDOP melalui matriks geometri H | Crazyflie + DWM1000 (Loco Positioning); laser altimeter VL53L1x | Ruang uji terbatas; objek uji berupa UAV | Bukti kuantitatif hubungan PDOP dan akurasi; anchor koplanar membuat galat sumbu z 116 cm |
| Yang, H. et al. (2024) | Ultra-Wideband Ranging Error Mitigation with Novel Channel Impulse Response Feature Parameters and Two-Step NLOS Identification | 2024 | Eksperimen (3 penghalang: manusia, dinding, kaca) + ML | Decision Tree + Feedforward Neural Network; fitur CIR baru (TFP delay, energy rise) | UWB dengan akses CIR | Memerlukan CIR; algoritma kompleks dan relatif lambat | Identifikasi NLOS 95,05%; peningkatan akurasi posisi hingga 54,46% |
| Hapsari, G.I., Erfianto, B., Munadi, R. & Irawati, I.D. (2025) | Modified Deffuant-Weisbuch Model for Anchor Node Distance Using Consensus-Base Self-Calibration | 2025 | Pemodelan matematis + simulasi konvergensi & stabilitas (proof-of-concept) | Modified Deffuant-Weisbuch (konsensus) + Random Forest (klasifikasi LOS/NLOS/Multipath); bukti konvergensi Lipschitz 0<μ<0.5 | Simulasi komputasi; tidak bergantung hardware UWB spesifik (konsep TWR antar-anchor) | Masih proof-of-concept (ranging, belum trilaterasi/posisi); dibatasi ruang bersisi sama; perlu optimasi untuk skala besar & real-time | Self-calibration tanpa bergantung satu anchor referensi; adaptif LOS/NLOS/Multipath; konvergen stabil 30–40 iterasi; potensial untuk ruangan banyak obstacle |
| Hapsari, G.I., Munadi, R., Erfianto, B. & Irawati, I.D. (2025) | Future Research and Trends in Ultra-Wideband Indoor Tag Localization | 2025 | Systematic Mapping Study (SMS) + Systematic Literature Review (SLR); bibliometrik & topic modeling | Bibliometrik VOSviewer + topic modeling LDA (Orange); PRISMA; taksonomi metrik (akurasi, skalabilitas, latensi, energi, biaya, kompleksitas) | Data 621 paper (Scopus/ScienceDirect/IEEE, 2017–2023); VOSviewer, Orange Data Mining | Berupa review/taksonomi, bukan sistem end-to-end; tidak menguji hardware/implementasi langsung | Memetakan tren & celah riset UWB indoor tag localization (LOS/NLOS/multipath, fusi sensor, RTLS); relevan sebagai peta arah pengembangan proyek |

DAFTAR PUSTAKA

[1] Krebs, S., & Herter, T. (2024). Ultra-Wideband (UWB) Positioning System Based on ESP32 and DWM3000 Modules. arXiv:2403.10194. https://arxiv.org/abs/2403.10194

[2] Shalihan, M., Liu, R., & Yuen, C. (2022). NLOS Ranging Mitigation with Neural Network Model for UWB Localization. arXiv:2206.09607. https://arxiv.org/abs/2206.09607

[3] Angarano, S., Mazzia, V., Salvetti, F., Fantin, G., & Chiaberge, M. (2021). Robust Ultra-wideband Range Error Mitigation with Deep Learning at the Edge. arXiv:2011.14684. https://arxiv.org/abs/2011.14684

[4] Fan, R., & Du, X. (2022). NLOS Error Mitigation Using Weighted Least Squares and Kalman Filter in UWB Positioning. arXiv:2205.05939. https://arxiv.org/abs/2205.05939

[5] Bregar, K. (2023). Indoor UWB Positioning and Position Tracking Data Set. Scientific Data, 10, 744. https://doi.org/10.1038/s41597-023-02639-5

[6] Yao, L., Yao, L., & Wu, Y.-W. (2021). Analysis and Improvement of Indoor Positioning Accuracy for UWB Sensors. Sensors, 21(17), 5731. https://doi.org/10.3390/s21175731

[7] Kramaric, L., Mustra, M., & Radisic, T. (2025). The Influence of Ultra-Wideband Anchor Placement on Localization Accuracy. Sensors, 25(16), 5115. https://doi.org/10.3390/s25165115

[8] Yang, H., Wang, Y., Xu, S., Bi, J., Jia, H., & Seow, C. K. (2024). Ultra-Wideband Ranging Error Mitigation with Novel Channel Impulse Response Feature Parameters and Two-Step Non-Line-of-Sight Identification. Sensors, 24(5), 1703. https://doi.org/10.3390/s24051703

[9] Hapsari, G. I., Erfianto, B., Munadi, R., & Irawati, I. D. (2025). Modified Deffuant-Weisbuch Model for Anchor Node Distance Using Consensus-Base Self-Calibration. International Journal of Intelligent Engineering and Systems, 18(5), 212–224. https://doi.org/10.22266/ijies2025.0630.16

[10] Hapsari, G. I., Munadi, R., Erfianto, B., & Irawati, I. D. (2025). Future Research and Trends in Ultra-Wideband Indoor Tag Localization. IEEE Access, 13, 21827–21852. https://doi.org/10.1109/ACCESS.2024.3399476