TABEL REVIEW JURNAL

Sistem Indoor Positioning Berbasis Ultra-Wideband (UWB) DW1000 pada ESP32

Nama: M Yusuf Rifa Afriza         NIM: Afriza25

| AUTHOR | JUDUL | TAHUN | METODA | ALGORITMA | ALAT | KEKURANGAN | KELEBIHAN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Krebs, S. & Herter, T. (2024) | Ultra-Wideband (UWB) Positioning System Based on ESP32 and DWM3000 Modules | 2024 | Rancang bangun / prototipe | Two-Way Ranging (TWR) + Extended Kalman Filter (EKF) di sisi tag | 6 board ESP32 + modul DWM3000 (Qorvo); 1 tag, 5 anchor | Jumlah anchor tetap; posisi anchor harus diketahui a priori; NLOS tidak dibahas khusus | Akurasi hingga 10 cm; EKF berjalan lokal di tag sehingga tidak bergantung server; arsitekturnya paling mirip dengan proyek ini |
| Shalihan, M., Liu, R. & Yuen, C. (2022) | NLOS Ranging Mitigation with Neural Network Model for UWB Localization | 2022 | Eksperimen + pembelajaran mesin | Neural Network (probabilitas LOS) + Weighted Least Square (WLS) | Modul UWB low-cost (tanpa data CIR) | Perlu data latih untuk setiap lingkungan; hanya memanfaatkan kuat sinyal | Akurasi naik 16,93% (lobi) dan 27,97% (koridor); tidak memerlukan CIR |
| Angarano, S. et al. (2021) | Robust Ultra-wideband Range Error Mitigation with Deep Learning at the Edge | 2021 | Deep learning + eksperimen | Representasi CIR (CNN) + optimasi graph | Transceiver UWB dengan akses CIR | Memerlukan CIR mentah; beban komputasi lebih besar | Koreksi galat NLOS/LOS di sisi edge; ringan untuk perangkat embedded |
| Fan, R. & Du, X. (2022) | NLOS Error Mitigation Using Weighted Least Squares and Kalman Filter in UWB Positioning | 2022 | Simulasi + eksperimen | WLS-RKF: uji hipotesis Mahalanobis + Kalman filter | Testbed UWB positioning | Perlu penyetelan ambang; bergantung pada model gerak | Tidak memerlukan pengetahuan awal distribusi NLOS; akurasi 5 cm |
| Bregar, K. (2023) | Indoor UWB Positioning and Position Tracking Data Set | 2023 | Pengukuran / penyusunan dataset | ADS-TWR, multilateration (least squares) sebagai baseline ML | DWM1000 + STM32F103 + Raspberry Pi 3; komunikasi MQTT | Hanya berupa dataset, bukan sistem real-time; memerlukan praproses | 8 anchor, 4 lingkungan indoor, menyertakan CIR + ground truth laser; memakai MQTT seperti proyek ini |
| Yao, L., Yao, L. & Wu, Y.-W. (2021) | Analysis and Improvement of Indoor Positioning Accuracy for UWB Sensors | 2021 | Model matematis + simulasi MATLAB + eksperimen | EKF; model galat 2D/3D; distribusi skew-t untuk kondisi NLOS | DWM1000 + STM32F0/F4 + IMU MPU9250 + Raspberry Pi 3 | Butuh minimal 3 anchor (2D) atau 4 anchor (3D); model cukup kompleks | Memberi panduan penempatan anchor optimal; EKF unggul saat data mengandung outlier |
| Kramaric, L., Mustra, M. & Radisic, T. (2025) | The Influence of Ultra-Wideband Anchor Placement on Localization Accuracy | 2025 | Eksperimen (4 konfigurasi anchor) | Trilateration DS-TWR + perhitungan PDOP melalui matriks geometri H | Crazyflie + DWM1000 (Loco Positioning); laser altimeter VL53L1x | Ruang uji terbatas; objek uji berupa UAV | Bukti kuantitatif hubungan PDOP dan akurasi; anchor koplanar membuat galat sumbu z 116 cm |
| Yang, H. et al. (2024) | Ultra-Wideband Ranging Error Mitigation with Novel Channel Impulse Response Feature Parameters and Two-Step NLOS Identification | 2024 | Eksperimen (3 penghalang: manusia, dinding, kaca) + ML | Decision Tree + Feedforward Neural Network; fitur CIR baru (TFP delay, energy rise) | UWB dengan akses CIR | Memerlukan CIR; algoritma kompleks dan relatif lambat | Identifikasi NLOS 95,05%; peningkatan akurasi posisi hingga 54,46% |
| Alarifi, A. et al. (2016) | Ultra Wideband Indoor Positioning Technologies: Analysis and Recent Advances | 2016 | Tinjauan pustaka (review) | Perbandingan ToA, TDoA, AoA, RSS; trilateration | Berbagai teknologi indoor positioning | Tanpa eksperimen langsung; referensi sudah cukup lama | Dasar teori kuat; membandingkan teknologi IPS secara menyeluruh |
| Kram, S. et al. (2019) | UWB Channel Impulse Responses for Positioning in Complex Environments: A Detailed Feature Analysis | 2019 | Analisis fitur + pembelajaran mesin | Ekstraksi fitur CIR + klasifikasi (SVM dan sejenisnya) | UWB DW1000 dengan data CIR | Berfokus pada analisis fitur, bukan sistem end-to-end | Panduan memilih fitur CIR untuk deteksi NLOS; analisisnya mendalam |
| Park, K. et al. (2020) | Multilateration under Flip Ambiguity for UAV Positioning Using Ultrawide-Band | 2020 | Simulasi + eksperimen | Multilateration; deteksi dan resolusi flip ambiguity (solusi cermin) | Anchor UWB + UAV | Berfokus pada UAV 3D; menuntut geometri anchor yang baik | Menjelaskan flip ambiguity, yaitu dua solusi cermin yang sangat relevan untuk kasus dua anchor |
| Zafari, F., Gkelias, A. & Leung, K. (2019) | A Survey of Indoor Localization Systems and Technologies | 2019 | Tinjauan pustaka (review) | Perbandingan AoA, ToF, RSSI, dan fingerprinting | WiFi, BLE, UWB, RFID, dan lainnya | Tidak khusus UWB; tanpa eksperimen | Taksonomi lengkap teknologi dan teknik lokalisasi indoor |
| Liu, C., Kadja, T. & Chodavarapu, V. P. (2022) | Experimental Evaluation of Sensor Fusion of Low-Cost UWB and IMU for Localization under Indoor Dynamic Testing Conditions | 2022 | Eksperimen (kondisi bergerak/dinamis) | Fusi sensor UWB + IMU (Kalman / komplementer) | Modul UWB low-cost + IMU | Memerlukan IMU tambahan; perlu kalibrasi | Evaluasi empiris fusi UWB-IMU saat tag bergerak |

DAFTAR PUSTAKA

[1] Krebs, S., & Herter, T. (2024). Ultra-Wideband (UWB) Positioning System Based on ESP32 and DWM3000 Modules. arXiv:2403.10194. https://arxiv.org/abs/2403.10194

[2] Shalihan, M., Liu, R., & Yuen, C. (2022). NLOS Ranging Mitigation with Neural Network Model for UWB Localization. arXiv:2206.09607. https://arxiv.org/abs/2206.09607

[3] Angarano, S., Mazzia, V., Salvetti, F., Fantin, G., & Chiaberge, M. (2021). Robust Ultra-wideband Range Error Mitigation with Deep Learning at the Edge. arXiv:2011.14684. https://arxiv.org/abs/2011.14684

[4] Fan, R., & Du, X. (2022). NLOS Error Mitigation Using Weighted Least Squares and Kalman Filter in UWB Positioning. arXiv:2205.05939. https://arxiv.org/abs/2205.05939

[5] Bregar, K. (2023). Indoor UWB Positioning and Position Tracking Data Set. Scientific Data, 10, 744. https://doi.org/10.1038/s41597-023-02639-5

[6] Yao, L., Yao, L., & Wu, Y.-W. (2021). Analysis and Improvement of Indoor Positioning Accuracy for UWB Sensors. Sensors, 21(17), 5731. https://doi.org/10.3390/s21175731

[7] Kramaric, L., Mustra, M., & Radisic, T. (2025). The Influence of Ultra-Wideband Anchor Placement on Localization Accuracy. Sensors, 25(16), 5115. https://doi.org/10.3390/s25165115

[8] Yang, H., Wang, Y., Xu, S., Bi, J., Jia, H., & Seow, C. K. (2024). Ultra-Wideband Ranging Error Mitigation with Novel Channel Impulse Response Feature Parameters and Two-Step Non-Line-of-Sight Identification. Sensors, 24(5), 1703. https://doi.org/10.3390/s24051703

[9] Alarifi, A., Al-Salman, A., Alsaleh, M., Alnafessah, A., Al-Hadhrami, S., Al-Ammar, M. A., & Al-Khalifa, H. S. (2016). Ultra Wideband Indoor Positioning Technologies: Analysis and Recent Advances. Sensors, 16(5), 707. https://doi.org/10.3390/s16050707

[10] Kram, S., Stahlke, M., Feigl, T., Seitz, J., & Thielecke, J. (2019). UWB Channel Impulse Responses for Positioning in Complex Environments: A Detailed Feature Analysis. Sensors, 19(24), 5547. https://doi.org/10.3390/s19245547

[11] Park, K., Kang, J., Arjmandi, Z., Shahbazi, M., & Sohn, G. (2020). Multilateration under Flip Ambiguity for UAV Positioning Using Ultrawide-Band. ISPRS Annals, V-1-2020, 317-324. https://doi.org/10.5194/isprs-annals-V-1-2020-317-2020

[12] Zafari, F., Gkelias, A., & Leung, K. K. (2019). A Survey of Indoor Localization Systems and Technologies. IEEE Communications Surveys & Tutorials, 21(3), 2568-2599. https://arxiv.org/abs/1709.01015

[13] Liu, C., Kadja, T., & Chodavarapu, V. P. (2022). Experimental Evaluation of Sensor Fusion of Low-Cost UWB and IMU for Localization under Indoor Dynamic Testing Conditions. Sensors, 22(21), 8156. https://doi.org/10.3390/s22218156